# ANxxxxx

Ethernet-to-Six-CAN FD Bridge Based on MCXE31B

### Rev. 0 - 04 July 2026

Document information

| Information | Content |
| --- | --- |
| Keywords | MCXE31B, Ethernet, CAN FD, SocketCAN, E2CF, eth2can |
| Abstract | This application note describes a bridge that exposes six MCXE31B CAN FD channels to a Linux host as standard SocketCAN devices over a raw Ethernet E2CF link. |

# Introduction

This application note describes an Ethernet-to-six-CAN FD bridge based on the MCXE31B microcontroller. The bridge connects a Linux host to six physical CAN FD channels through a raw Layer-2 Ethernet transport named E2CF. On Linux, the eth2can kernel module exposes the gateway as standard SocketCAN network devices, eth2can0 through eth2can5.

The document focuses on customer bring-up: hardware connections, firmware and driver roles, installation, basic traffic checks, performance test entry points, and troubleshooting. The implementation notes are limited to details that help users understand behavior and diagnose integration issues.

# System overview

Figure 1. MCXE31B Ethernet-to-six-CAN FD bridge overview

The Linux host does not need a CAN controller for each bus. Instead, it loads eth2can.ko, binds the module to an Ethernet interface, and exchanges E2CF frames with the MCXE31B gateway. The gateway firmware receives and transmits CAN FD frames on FlexCAN0 through FlexCAN5 and moves them over ENET_QOS raw Ethernet frames without an IP stack.

| Item | Value used by this delivery |
| --- | --- |
| Linux CAN interface | SocketCAN devices eth2can0 through eth2can5 |
| Default CAN FD rate | Nominal 1 Mbit/s, data phase 5 Mbit/s, BRS enabled |
| Gateway transport | Raw Ethernet, EtherType 0x88B5; VLAN VID 100 supported for deployment |
| Default bring-up frame mode | Untagged E2CF frames, peer MAC learned from gateway heartbeat |
| Customer test tool | linux/can_testcase/canperf for latency and bandwidth verification |

## Bridge operating model

The gateway is a Layer-2 bridge for CAN FD traffic, not a CANopen, J1939, or application-protocol gateway. CAN identifiers, DLC, BRS state, payload, and error-state indicators are carried through the E2CF DATA records and exposed on Linux through the standard SocketCAN API. This allows existing Linux CAN tools and applications to run without a dedicated CAN controller on the host.

Each E2CF channel maps one-to-one to one FlexCAN instance and one SocketCAN network device. Per-channel transmit order is preserved by using one active FlexCAN transmit message buffer and a 16-slot host echo window. Ordering between different CAN channels is intentionally independent, matching the behavior of six separate physical CAN controllers.

| Bridge aspect | Behavior visible to the customer |
| --- | --- |
| Host model | Six SocketCAN devices, eth2can0 through eth2can5, on one Ethernet interface |
| Transport model | Raw Ethernet frames with EtherType 0x88B5; VLAN VID 100 can be enabled for deployment |
| CAN model | CAN FD BRS by default at 1 Mbit/s nominal and 5 Mbit/s data phase |
| Ordering model | Per-channel ordering is preserved; channels run independently |
| Acceptance model | Use RESULT and EVIDENCE lines from canperf plus clean driver/gateway counters |

# Hardware setup

The MCXE31B board connects one RMII Ethernet PHY and six external CAN FD transceivers. Use CAN SIC-grade or otherwise application-appropriate CAN FD transceivers for the external CAN buses. Each physical CAN bus requires correct transceiver power, a common ground reference, correct CANH/CANL polarity, and 120 ohm termination at both ends of the bus.

| Function | MCXE31B signals used in this project |
| --- | --- |
| CAN0 | FLEXCAN_0_TX PTA7 pin 100; FLEXCAN_0_RX PTA6 pin 102 |
| CAN1 | FLEXCAN_1_TX PTA11 pin 160; FLEXCAN_1_RX PTA12 pin 159 |
| CAN2 | FLEXCAN_2_TX PTE24 pin 157; FLEXCAN_2_RX PTE25 pin 158 |
| CAN3 | FLEXCAN_3_TX PTC28 pin 96; FLEXCAN_3_RX PTC29 pin 99 |
| CAN4 | FLEXCAN_4_TX PTC30 pin 101; FLEXCAN_4_RX PTC31 pin 103 |
| CAN5 | FLEXCAN_5_TX PTC27 pin 93; FLEXCAN_5_RX PTC26 pin 91 |
| Ethernet RMII | MDIO PTB4, MDC PTB5, TXD0 PTC2, TXD1 PTD7, TX_EN PTD12, RX_DV PTC17, RXD0 PTC1, RXD1 PTC0, REF_CLK PTD11, PHY reset PTC3 |
| Debug UART | LPUART_5_RX PTE3 pin 27; LPUART_5_TX PTE14 pin 26 |

The default loopback harness used by the repository test flow connects three independent CAN FD buses:

eth2can0 <-> eth2can4

eth2can1 <-> eth2can2

eth2can3 <-> eth2can5

| LED | GPIO | State | Meaning |
| --- | --- | --- | --- |
| SYS | PTC16 | Blinking about 1 Hz | Firmware superloop is running |
| SYS | PTC16 | On | Firmware stopped in a fault or fatal path |
| NET | PTB22 | Off | PHY link is down |
| NET | PTB22 | Blinking | PHY link is up, but the E2CF peer heartbeat is not ready |
| NET | PTB22 | On | Ethernet and E2CF link are ready |
| CAN | PTC14 | Blinking | CAN RX or TX traffic is present |
| CAN | PTC14 | On | At least one active CAN channel is error-passive or bus-off |

## Hardware integration checklist

For customer hardware bring-up, verify the board at the physical layer before investigating protocol behavior. The E2CF link can be healthy while a single CAN bus still fails because of termination, polarity, transceiver supply, or ground-reference issues.

| Area | Check before software debugging |
| --- | --- |
| Ethernet PHY | RMII clock, MDIO/MDC access, PHY reset pin, link partner, and selected Linux --ifname |
| CAN transceivers | Transceiver power, common ground, CANH/CANL polarity, standby pins, and 120 ohm termination at both bus ends |
| Default harness | Use the validated pairs 0<->4, 1<->2, and 3<->5 before moving to customer wiring |
| VLAN path | For switched deployments, pass EtherType 0x88B5 and VLAN VID 100 without filtering |
| LEDs and UART | Use SYS/NET/CAN LED states and the debug UART as the first fault-localization signals |

# Software architecture

## MCXE31B firmware

The firmware is a bare-metal Cortex-M7 application. It does not use an RTOS or lwIP. CAN and Ethernet traffic are handled by interrupt paths; the main loop performs bounded housekeeping such as aggregation flush checks, heartbeat, event, time, statistics messages, PHY link polling, LED updates, CAN error polling, and deferred debug-log drain.

The firmware start sequence is BOARD_InitHardware, status LED initialization, gateway time initialization, debug logging, SysTick configuration, raw Ethernet initialization, CAN hardware initialization, E2CF core initialization, and then the superloop.

| Firmware resource | Configuration |
| --- | --- |
| FlexCAN instances | Six instances, FlexCAN0 through FlexCAN5 |
| FlexCAN PE clock | AIPS_PLAT_CLK at 80 MHz for the active gateway configuration |
| CAN receive resources | CAN0/CAN1/CAN2 use 8 RX MBs; CAN3/CAN4/CAN5 use 4 RX MBs |
| CAN transmit resources | One active TX message buffer per channel, preserving per-channel order |
| Software TX FIFO | 16 entries per channel, matching the E2CF window depth |
| ENET_QOS resources | 16 RX descriptors, 64 TX descriptors, 8 TX staging faces |
| Interrupt priorities | CAN and EMAC use priority 1; SysTick uses priority 3 |

## Linux driver

The Linux driver module is named eth2can. It binds to one lower Ethernet interface and creates six SocketCAN network devices. The module defaults to ifname=eth0, vid=-1 for untagged bring-up, and an empty peer parameter so the driver can learn the MCXE31B peer MAC from heartbeat frames.

Each SocketCAN TX skb is validated, assigned a free echo slot, encoded as an E2CF DATA frame, and sent on the lower Ethernet interface. The echo slot is released only when the MCU returns a TXC record after the CAN frame completes on the CAN bus. If the 16-slot per-channel window fills, the driver stops that channel queue until TXC records free slots.

## E2CF protocol

Figure 2. E2CF data, completion, and management paths

E2CF is a little-endian, packed, raw Ethernet protocol shared by the MCU firmware and the Linux driver. The two e2cf_proto.h copies in source/ and linux/src/ are byte-identical and define the wire format. DATA carries CAN frames or transmit requests. TXC returns transmit completion status. EVT, CFG_REQ, CFG_RSP, TIME, HB, and STATS provide management, recovery, and diagnostics.

| Protocol item | Value |
| --- | --- |
| Version | 1 |
| EtherType | 0x88B5 |
| VLAN | VID 100 supported for deployment; untagged frames allowed during bring-up |
| PCP usage | PCP 6 for DATA/TXC/EVT/TIME/HB; PCP 2 for CFG_REQ/CFG_RSP/STATS |
| Window depth | 16 transmit slots per channel |
| MCU-to-Linux aggregation | Records are sent when full or when the 50 us aggregation timer expires |
| Heartbeat | 100 ms period; MCU enters safe state after 500 ms without Linux heartbeat |
| Time base | Gateway monotonic time from the Cortex-M7 DWT cycle counter, reported through TIME messages |

## Traffic handling and recovery

The Linux driver applies normal SocketCAN backpressure when the 16-slot transmit window for a channel is full. The queue is restarted when TXC records arrive from the gateway. A TXC record is returned after CAN bus completion, so this mechanism covers the Linux driver, Ethernet transport, firmware queueing, and FlexCAN transmission path.

MCU-to-Linux DATA and TXC records are aggregated by record count or by the 50 us aggregation timer. Aggregation reduces Ethernet frame overhead while preserving record sequence information. The Linux driver also tracks receive sequence gaps and distinguishes confirmed loss from in-window reordering before reporting dirty counters.

| Condition | Expected bridge behavior | Primary evidence |
| --- | --- | --- |
| Heartbeat present | Gateway peer is learned and SocketCAN carrier can remain on | NET LED on, dmesg gateway-alive messages, EtherType 0x88B5 HB frames |
| Heartbeat timeout | Linux carrier is turned off and the gateway enters a safe state for host-driven traffic | 500 ms timeout path, NET LED not ready |
| CAN bus-off or error-passive | Channel state is reported through EVT/STATS and SocketCAN state handling | CAN LED on, ethtool/debugfs counters, dmesg |
| Gateway reboot | Driver detects reboot and schedules channel recovery instead of silently continuing stale state | reboot recovery log and renewed CFG sequence |
| Window pressure | SocketCAN TX queue is stopped until TXC frees echo slots | No fixed performance threshold; inspect counters and canperf evidence |

# Bring-up procedure

Build or confirm the MCXE31B gateway firmware. If the board is not preprogrammed, open e2cf_mcxe31.uvprojx in Keil MDK, build the firmware, and program the device through the normal MCXE31B/SWD flow.

Connect the Linux host Ethernet interface to the gateway Ethernet port and power the MCXE31B board.

Install and load the Linux driver from the repository root:

sh linux/scripts/install_driver.sh

Check that six SocketCAN devices exist and are configured for CAN FD 1M/5M:

ip -details link show type can

Run a basic receive/transmit check on one default harness pair:

candump eth2can4 &

cansend eth2can0 123##3001122334455667788

If the receive side sees the frame, the Linux driver, raw Ethernet link, gateway firmware, CAN transceivers, and one physical CAN FD bus are all functioning for basic traffic.

## Bring-up acceptance checkpoints

Use a staged bring-up sequence. Do not start with a bandwidth stress run until the Ethernet link, heartbeat, SocketCAN configuration, and at least one physical CAN pair have passed a basic traffic check.

| Step | Pass condition |
| --- | --- |
| Driver load | eth2can0..5 are registered and dmesg does not report module-load errors |
| Heartbeat | The driver observes the gateway heartbeat on the selected Ethernet interface and VLAN setting |
| CAN configuration | All six devices accept CAN FD 1M/5M configuration and transition to up state |
| Basic traffic | cansend on one side of a default pair is visible with candump on the peer side |
| Latency | canperf latency --count 10000 reports PASS and an EVIDENCE line |
| Bandwidth | canperf bandwidth final confirmation reports PASS, MSR, and counters=clean |

When a run fails, keep the final RESULT and EVIDENCE lines together with dmesg, tcpdump heartbeat capture, and debugfs or ethtool statistics. This gives support teams enough context to separate host, Ethernet, firmware, and CAN physical-layer causes.

# Verification with canperf

The canperf utility is the customer-facing verification entry point. It runs over the SocketCAN devices and therefore verifies the Linux driver, Ethernet transport, MCXE31B firmware, and physical CAN FD buses together. Build it on the target Linux host:

cd linux/can_testcase

make

Run the latency test and the bidirectional maximum sustainable bandwidth test:

./canperf latency --count 10000

./canperf bandwidth

Use the final RESULT line as the customer-facing conclusion. The latency command reports p50, p99, and p99.9 end-to-end latency. The bandwidth command reports MSR, the maximum sustainable rate found by the final confirmation run. This application note intentionally does not specify fixed p99 or MSR thresholds; acceptance thresholds depend on the host, wiring, bus loading, and customer system constraints.

The EVIDENCE line is intended for repeatable records. A clean run reports zero_loss=yes and counters=clean, meaning the application did not detect lost frames and the selected driver/gateway loss, overflow, reject, starvation, and send-failure counters did not increase during the confirmation window.

# Troubleshooting

| Symptom | Checks |
| --- | --- |
| No eth2can0..5 devices | Check dmesg for module load errors and confirm CONFIG_CAN, CONFIG_CAN_RAW, and CONFIG_CAN_DEV are enabled in the running kernel. |
| Kernel build directory missing | Install headers for the exact running kernel or rerun with KDIR=/path/to/kernel/build. |
| No gateway heartbeat | Check cable, selected --ifname, switch/VLAN path, firmware image, and tcpdump for EtherType 0x88B5 heartbeat traffic. |
| CAN frame not received | Check default harness pairs, 120 ohm termination at both bus ends, CANH/CANL polarity, transceiver power, and common ground. |
| CAN LED on | At least one active channel is error-passive or bus-off. Check rate, wiring, termination, polarity, and transceiver supply. |
| counters=dirty | Use seq_lost, rx_ovf, rejects, face_starved, send_fail, and emac_rxdrop deltas to locate the failing path. |

A heartbeat capture command for bring-up is:

sudo tcpdump -eni eth0 'ether proto 0x88b5 or (vlan and ether proto 0x88b5)'

# Acronyms and abbreviations

| Term | Meaning |
| --- | --- |
| BRS | Bit Rate Switch, CAN FD data phase at a higher bit rate |
| CAN FD | Controller Area Network Flexible Data-rate |
| E2CF | Ethernet-to-CAN FD raw Ethernet protocol used by this gateway |
| EVT | E2CF event message for CAN state and error reporting |
| HB | E2CF heartbeat message for peer discovery and safe-state supervision |
| MSR | Maximum sustainable rate reported by canperf bandwidth |
| RMII | Reduced Media Independent Interface |
| SocketCAN | Linux CAN networking subsystem |
| TXC | Transmit completion message returned by the MCU after CAN bus completion |

# Revision history

Table 1 summarizes revisions to this document.

Table 1. Revision history

| Document ID | Release date | Description |
| --- | --- | --- |
| ANxxxxx v.0 | 04 July 2026 | Initial draft for review |

# Legal information

## Definitions

Draft — A draft status on a document indicates that the content is still under internal review and subject to formal approval, which may result in modifications or additions. NXP Semiconductors does not give any representations or warranties as to the accuracy or completeness of information included in a draft version of a document and shall have no liability for the consequences of use of such information.

## Disclaimers

Limited warranty and liability — Information in this document is believed to be accurate and reliable. However, NXP Semiconductors does not give any representations or warranties, expressed or implied, as to the accuracy or completeness of such information and shall have no liability for the consequences of use of such information. NXP Semiconductors takes no responsibility for the content in this document if provided by an information source outside of NXP Semiconductors.

In no event shall NXP Semiconductors be liable for any indirect, incidental, punitive, special or consequential damages (including - without limitation - lost profits, lost savings, business interruption, costs related to the removal or replacement of any products or rework charges) whether or not such damages are based on tort (including negligence), warranty, breach of contract or any other legal theory.

Notwithstanding any damages that customer might incur for any reason whatsoever, NXP Semiconductors’ aggregate and cumulative liability towards customer for the products described herein shall be limited in accordance with the Terms and conditions of commercial sale of NXP Semiconductors.

Right to make changes — NXP Semiconductors reserves the right to make changes to information published in this document, including without limitation specifications and product descriptions, at any time and without

notice. This document supersedes and replaces all information supplied prior to the publication hereof.

Suitability for use — NXP Semiconductors products are not designed, authorized or warranted to be suitable for use in life support, life-critical or safety-critical systems or equipment, nor in applications where failure or malfunction of an NXP Semiconductors product can reasonably be expected to result in personal injury, death or severe property or environmental damage. NXP Semiconductors and its suppliers accept no liability for inclusion and/or use of NXP Semiconductors products in such equipment or applications and therefore such inclusion and/or use is at the customer’s own risk.

Applications — Applications that are described herein for any of these products are for illustrative purposes only. NXP Semiconductors makes no representation or warranty that such applications will be suitable for the specified use without further testing or modification.

Customers are responsible for the design and operation of their applications and products using NXP Semiconductors products, and NXP Semiconductors accepts no liability for any assistance with applications or customer product design. It is customer’s sole responsibility to determine whether the NXP Semiconductors product is suitable and fit for the customer’s applications and products planned, as well as for the planned

application and use of customer’s third party customer(s). Customers should provide appropriate design and operating safeguards to minimize the risks associated with their applications and products.

NXP Semiconductors does not accept any liability related to any default, damage, costs or problem which is based on any weakness or default in the customer’s applications or products, or the application or use by customer’s third party customer(s). Customer is responsible for doing all

necessary testing for the customer’s applications and products using NXP Semiconductors products in order to avoid a default of the applications and the products or of the application or use by customer’s third party customer(s). NXP does not accept any liability in this respect.

Terms and conditions of commercial sale — NXP Semiconductors products are sold subject to the general terms and conditions of commercial sale, as published at https://www.nxp.com/profile/terms, unless otherwise agreed in a valid written individual agreement. In case an individual agreement is concluded only the terms and conditions of the respective agreement shall apply. NXP Semiconductors hereby expressly objects to applying the customer’s general terms and conditions with regard to the purchase of NXP Semiconductors products by customer.

Export control — This document as well as the item(s) described herein may be subject to export control regulations. Export might require a prior authorization from competent authorities.

Suitability for use in non-automotive qualified products — Unless this document expressly states that this specific NXP Semiconductors product is automotive qualified, the product is not suitable for automotive

use. It is neither qualified nor tested in accordance with automotive testing or application requirements. NXP Semiconductors accepts no liability for inclusion and/or use of non-automotive qualified products in automotive equipment or applications.

In the event that customer uses the product for design-in and use in automotive applications to automotive specifications and standards, customer (a) shall use the product without NXP Semiconductors’ warranty of the product for such automotive applications, use and specifications, and

(b) whenever customer uses the product for automotive applications beyond NXP Semiconductors’ specifications such use shall be solely at customer’s own risk, and (c) customer fully indemnifies NXP Semiconductors for any liability, damages or failed product claims resulting from customer design and use of the product for automotive applications beyond NXP Semiconductors’ standard warranty and NXP Semiconductors’ product specifications.

Translations — A non-English (translated) version of a document, including the legal information in that document, is for reference only. The English version shall prevail in case of any discrepancy between the translated and English versions.

Security — Customer understands that all NXP products may be subject to unidentified vulnerabilities or may support established security standards or specifications with known limitations. Customer is responsible for the design and operation of its applications and products throughout their lifecycles

to reduce the effect of these vulnerabilities on customer’s applications and products. Customer’s responsibility also extends to other open and/or proprietary technologies supported by NXP products for use in customer’s

applications. NXP accepts no liability for any vulnerability. Customer should regularly check security updates from NXP and follow up appropriately.

Customer shall select products with security features that best meet rules, regulations, and standards of the intended application and make the ultimate design decisions regarding its products and is solely responsible for compliance with all legal, regulatory, and security related requirements

concerning its products, regardless of any information or support that may be provided by NXP.

NXP has a Product Security Incident Response Team (PSIRT) (reachable at PSIRT@nxp.com) that manages the investigation, reporting, and solution release to security vulnerabilities of NXP products.

NXP B.V. — NXP B.V. is not an operating company and it does not distribute or sell products.

## Trademarks

Notice: All referenced brands, product names, service names, and trademarks are the property of their respective owners.

NXP — wordmark and logo are trademarks of NXP B.V.

Ethernet-to-Six-CAN FD Bridge Based on MCXE31B

Please be aware that important notices concerning this document and the product(s) described herein, have been included in section 'Legal information'.

© 2024 NXP B.V.	All rights reserved.

For more information, please visit: https://www.nxp.com

Date of release: 04 July 2026 Document identifier: ANxxxxx